reverse magnetic polarity

show magnetic field lines from magnet

show graphs for magnet and/or coil (solenoid)

N

N

Bonus! For exploring other scenarios!

view from bottom

vertical/horizontal

Zoom!

show motion of electrons in solenoid

various graphs!

Data analysis!! Really cool tool to help analyse voltage

Download the simulation ejs_FallingMagnet08.jar from the following link:

https://dl.dropbox.com/u/44365627/lookangEJSworkspace/export/ejs_FallingMagnet08.jar

OR	http://tinyurl.com/magfall				QR code on the right. (Needs Java to run.)

Universal symbols

Play / Start

Play in slow-mo i.e. step-by-step or frame-by-frame simulation

Reset to default values or factory settings. Click this after playing every simulation and before changing any settings.

Parameters (refer to bottom of the simulation) for exploration

| Parameter / unit | Definition/Meaning |
| --- | --- |
| Magnet_Z / m | Initial vertical position (z-axis) of the magnet |
| L / m (in red for magnet) | Length of magnet |
| B / T | Magnetic Field Strength of magnet |
| v / m/s (in red for magnet) | Initial speed of magnet |
| Solenoid_Z / m | Initial vertical position (z-axis) of the solenoid |
| L / m (in orange for solenoid) | Length of the solenoid |
| 30/L | This is meant to be number of turns per unit length. Due to programming constraints, it is 30 turns/Length set by user. |
| v / m/s (in orange for solenoid) | Velocity of solenoid (for exploration of relative velocity. ) |

| Scenario A – Falling Magnet through a Solenoid |  |
| --- | --- |
| Default Settings Parameter / unit; Magnitude. Magnet_Z / m; 0.500. L / m (in red for magnet); 0.100. B / T; 2.470. v / m/s (in red for magnet); 0.000. Solenoid_Z / m; –0.500. L / m (in orange for solenoid); 0.000 0.500. 30/L. v / m/s (in orange for solenoid); 0.000. - Change solenoid length to 0.500 m. - Must press Enter key. Please note that 30/L becomes 60.0 automatically. - Click on emf vs t (bottom right). - Click on Coil (bottom right above v). - Click to run the simulation. - Copy the simulated graph onto the axes provided. - Explain the shape of the simulated graph. | t E |
| ……………………………………………………………………………………………………………………… |  |
| ……………………………………………………………………………………………………………………… |  |
| ……………………………………………………………………………………………………………………… |  |
| ……………………………………………………………………………………………………………………… |  |
| ……………………………………………………………………………………………………………………… |  |
| ……………………………………………………………………………………………………………………… |  |
| ……………………………………………………………………………………………………………………… |  |